DNRME Water Monitoring Information

Hydrology Archive Data Information Sheet

Content/ Resources

DNRME supplies Time Series water monitoring data as compressed ZIP archives. The ZIP archives are:

| DNRME_SDVR_OPEN.ZIP | Time Series Stage, Discharge, Volume, Rainfall for Open Gauging Stations |
| --- | --- |
| DNRME_SDVR_CLOSED.ZIP | Time Series Stage, Discharge, Volume, Rainfall for Closed Gauging Stations |
| DNRME_TSQW_OPEN.ZIP | Time Series Water Quality for… |
| DNRME_TSQW_CLOSED.ZIP | Time Series Water Quality |
| DNRME_RAIN.ZIP | Time Series Rainfall for Open and Closed Stations |
| DNRME_ART.ZIP | Time Series Artesian Level for Open and Closed Stations |
| DNRME_SUBART.ZIP | Time Series Groundwater Level for Open and Closed Stations |

Metadata and File Types

- Metadata Files

| Surface Water Monitoring Network Index | WMP016_8.0_Stream_Gauging_Station_Network_2019_20.pdf |
| --- | --- |
| Groundwater Monitoring Network Index | WMP017_7.0_Groundwater_Network_2019_20.pdf |
| Rainfall index |  |
| [Separate CSVs for the indexes] | Spatial data for what ever it is |
| Quality Code Mapping | DNRME_DA_QUALITY.CSV |
| DNRME Water Monitoring Data Information | Hydrology Archive Data Information Sheet (this document) |
| Index of individual data extracts |  |
| [GS].SR.TXT | Individual Gauging Station Summary Report |

CSV Data File Type: [GS].[TYPE].CSV

[GS]  Abbreviation for "Gauging Station", also referred to as a "Site".

[SS]  Abbreviation for "Subartesian Station", also referred to as a "Site".

[AS]  Abbreviation for "Artesian Station", also referred to as a "Site".

- Point Hydrologic Data

| PS | Point Stage (i.e. 'Gauge Height') as measured |
| --- | --- |
| PD | Point Discharge as stored * |
| SS | Point Stage (i.e. 'Subartesian Water Level') as measured |
| AS | Point Stage (i.e. 'Artesian EPS') as measured |
| PR | Point Rainfall as measured |

.

* Note that DNRME manages Time Series Stage and Discharge records independently once the latter has been calculated via the rating process. This allows, for example and only in certain circumstances, continuous discharge records to be maintained even when periods of stage data may be missing due to instrument malfunction or damage and the discharge is able to be estimated and also verified by competent and qualified hydrographic staff

- Periodic Hydrologic Extracts (nb. START/ END indicates the period to which each record refers)

| DM | [GS] Daily Midnight Stage (min, max, mean), Discharge (min, max, mean), Volume (total), Rainfall (total),START |
| --- | --- |
| SM | [SS] Daily Midnight Water Level (min, max, mean), Rainfall (total),START |
| AM | [AS] Daily Midnight EPS (min, max, mean), Rainfall (total),START |
| D9 | [GS] Daily 0900 Stage (min, max, mean), Discharge (min, max, mean), Volume (total), Rainfall (total), END |
| S9 | [SS] Daily 0900 Water Level (min, max, mean), Rainfall (total), END |
| A9 | [AS] Daily 0900 EPS (min, max, mean), Rainfall (total), END |
| DRM | Daily Midnight Rainfall (total), START |
| DR9 | Daily 0900 Rainfall (total), END |

- Point Time Series Water Quality (TSWQ)

| PWQ | Point Water Quality- as measured EC, Temperature, pH, Turbidity, Instantaneous Stage and Discharge |
| --- | --- |

- Periodic Time Series Water Quality Extracts

| DET | Daily Midnight EC (min, max, mean), Temperature (min, max, mean), START |
| --- | --- |
| DPT | Daily Midnight pH (min, max, mean), Temperature (min, max, mean), START |
| DTU | Daily Midnight Turbidity (min, max, mean), START |

- Time Series Data Parameters, Units

| Name | Description | Variable Number |
| --- | --- | --- |
| Stage | m GHt (metres Gauge Height) | 100.00 |
| Discharge | m3/sec (cubic metres per second, aka 'cumecs') | 140.00 |
| Volume | ML (megalitres) | 151.00 |
| Rainfall | mm (millimetres) | 10.00 |
| Electrical Conductivity (EC) | μS/cm (micro Siemens per centimetre) | 2010.00 |
| Temperature | °C (degrees Celsius) | 2080.00 |
| pH | pH (pH units) | 2100.00 |
| Turbidity | NTU (Nephelometric Turbidity Units) | 2030.00 |
| Bore Water Level | m (metres) | 110.00 |
| Groundwater Elevation AHD | m (metres) | 111.00 |
| Artesian Equivalent Potentiometric Surface (EPS) | m (metres) | 113.00 |
| Artesian Equivalent Potentiometric Elevation AHD (EPS) | m (metres) | 114.00 |

formatting

Copyright © The State of Queensland 2020 The Queensland Government supports and encourages the distribution of its material. Unless otherwise noted, all copyright material available on or through this website is licensed under a Creative Commons Attribution 4.0 International licence (CC BY 4.0).

[image with hyperlink]

You are free to use copyright material available on or through this website that is covered by a CC BY licence in line with the licence terms. You must keep the copyright notice on the copyright material and attribute the State of Queensland as the source of the copyright material. Note: Creative Commons licences relate to copyright material, not to other forms of intellectual property. Some material accessible on or through this website may not be available under a Creative Commons licence. Website material not available under a Creative Commons licence  the government coat of arms or government logos  trade mark protected material  intellectual property (including copyright) owned by third parties  material available under other specified licences. Linked material available on other Queensland Government websites A user of this website who uses the links provided to another Queensland Government agency’s website and material available on or through there, acknowledges and agrees that the terms of use, including licence terms, set out on the other agency’s website apply to the use that may be made of those materials. Disclaimer As far as practicable, material for which the copyright is owned by a third party will be clearly labelled. The Queensland Government has made all reasonable efforts to ensure that this material has been reproduced with the consent of the copyright owners. Contact us if copyright information is missing from material you wish to reuse.