Department of Regional Development, Manufacturing and Water

Water Monitoring Information Hydrology Archive Data Information Sheet

This publication has been compiled by Divisional Support, Department of Regional Development Manufacturing and Water.

© State of Queensland, 2021

The Queensland Government supports and encourages the dissemination and exchange of its information. The copyright in this publication is licensed under a Creative Commons Attribution 4.0 International (CC BY 4.0) licence.

Under this licence you are free, without having to seek our permission, to use this publication in accordance with the licence terms. You must keep intact the copyright notice and attribute the State of Queensland as the source of the publication.

Note: Some content in this publication may have different licence terms as indicated.

For more information on this licence, visit https://creativecommons.org/licenses/by/4.0/.

The information contained herein is subject to change without notice. The Queensland Government shall not be liable for technical or other errors or omissions contained herein. The reader/user accepts all risks and responsibility for losses, damages, costs and other consequences resulting directly or indirectly from using this information.

Interpreter statement:

The Queensland Government is committed to providing accessible services to Queenslanders from all culturally and linguistically diverse backgrounds. If you have difficulty in understanding this document, you can contact us within Australia on 13QGOV (13 74 68) and we will arrange an interpreter to effectively communicate the report to you.

Contents

1.0	Introduction	2

2.0	Validation	2

3.0	Content/ Resources	3

4.0	Metadata and File Types	3

4.1	Metadata Files	3

5.0	Index of individual data extracts	4

5.1	Point Hydrologic Data	4

5.2	Periodic Hydrologic Extracts	4

5.3	Point Time Series Water Quality (TSWQ)	5

5.4	Periodic Time Series Water Quality Extracts	5

6.0	Time Series Data Parameters, Units	5

7.0	Copyright	6

8.0	Disclaimer	6

# Introduction

The Queensland Department of Regional Development, Manufacturing and Water (the Department) operates surfacewater monitoring networks under an approved Water Monitoring Business Framework and ISO 9001:2015 accredited Quality Management System. In accordance with the Water Monitoring Business Framework, information collected from gauging stations is primarily stream height, flow and volume. These data can be accessed as:

Unverified telemetry data which can be viewed on the Department’s Water Monitoring Information Portal. This information is routinely published in near- real time following automated screening for any significant inconsistencies:

Website 	https://water-monitoring.information.qld.gov.au/

Validated historic streamflow data can also be downloaded from the Department’s Water Monitoring Information Portal. This information is validated by qualified hydrographic staff and is quality coded accordingly. Validated historic data can also be obtained by formal request to the Department:

Email 	water.monitoring@dnrm.qld.gov.au

Post 	Water Monitoring and Reporting

Divisional Support

Department of Regional Development, Manufacturing and Water

PO Box 15216, City East, Qld 4002

# Validation

The Department endeavours to validate data as quickly as possible, while the performance indicator aims for data to be validated within 220 days. You may download current validated data periodically from the Open Data Portal.

With respect to use of data:

Unvalidated data viewed on the Department’s Water Monitoring Information Portal has not been rigorously assessed and the data is quality coded to indentify this. Unvalidated data should by used with care as they may change after validation. These data should only be used by persons who are familiar with the characteristics of streamflow information.

Validated data are data that has been assessed and is the best available quality at the time, however the data should always be interpreted taking into account the quality codes that have been applied.

Hydrologic advice should be sought to assist with any data interpretation.

# Content/ Resources

The Department supplies Time Series water monitoring data as compressed ZIP archives. The ZIP archives are:

| FILE NAME | DESCRIPTION |
| --- | --- |
| RDMW_SDVR_OPEN.ZIP | Time Series Stage, Discharge, Volume, Rainfall for Open Gauging Stations |
| RDMW_SDVR_CLOSED.ZIP | Time Series Stage, Discharge, Volume, Rainfall for Closed Gauging Stations |
| RDMW_TSWQ_OPEN.ZIP | Time Series Water Quality for Open Gauging Stations |
| RDMW_TSWQ_CLOSED.ZIP | Time Series Water Quality Closed Gauging Stations |
| RDMW_RAIN.ZIP | Time Series Rainfall for Open and Closed Stations |
| RDMW_ART.ZIP | Time Series Artesian Equivalent Level for Open and Closed Stations |
| RDMW_SUBART.ZIP | Time Series Groundwater Level for Open and Closed Stations |

# Metadata and File Types

## Metadata Files

| FILE NAME | DESCRIPTION |
| --- | --- |
| Stream_Gauging_Station_Network_2019-20.PDF | Surface Water Monitoring Network Index |
| Groundwater_Network_2019-20.PDF | Groundwater Monitoring Network Index |
| Rainfall_Site_Network_2019-20.PDF | Rainfall index |
| Surface_Water_Ambient_Network_2019-20.PDF | Surface Water Quality Network Index |
| Stream_Gauging_Station_Network_2019-20.CSV | Spatial data for Stream Gauging Station Network |
| Groundwater_Network_2019-20.CSV | Spatial data for Groundwater Network |
| Rainfall_Site_Network_2019-20.CSV | Spatial data for Rainfall Site Network |
| Surface_Water_Ambient_Network_2019-20.CSV | Surface Water Quality Network |
| RDMW_DA_QUALITY.CSV | Quality Code Mapping |
| RDMW_WMIS-Resource_Catalogue.PDF | Hydrology Archive Resource Catalogue |
| RDMW_WMIS–Information_Sheet.PDF | Hydrology Archive Data Information Sheet (this document) |

# Index of individual data extracts

CSV Data File Type: [GS].[TYPE].CSV

[GS]  Abbreviation for "Gauging Station", also referred to as a "Site".

[SS]  Abbreviation for "Subartesian Station", also referred to as a "Site".

[AS]  Abbreviation for "Artesian Station", also referred to as a "Site".

## Point Hydrologic Data

| PS | Point Stage (i.e. 'Gauge Height') as measured |
| --- | --- |
| PD | Point Discharge as stored * |
| SS | Point Stage (i.e. 'Subartesian Equivalent Water Level') as measured |
| AS | Point Stage (i.e. 'Artesian EPS') as measured |
| PR | Point Rainfall as measured |

## Periodic Hydrologic Extracts

(nb. START/ END indicates the period to which each record refers)

| DM | [GS] Daily Midnight Stage (min, max, mean), Discharge (min, max, mean), Volume (total), Rainfall (total),START |
| --- | --- |
| SM | [SS] Daily Midnight Water Level (min, max, mean), Rainfall (total),START |
| AM | [AS] Daily Midnight EPS (min, max, mean), Rainfall (total),START |
| D9 | [GS] Daily 0900 Stage (min, max, mean), Discharge (min, max, mean), Volume (total), Rainfall (total), END |
| S9 | [SS] Daily 0900 Water Level (min, max, mean), Rainfall (total), END |
| A9 | [AS] Daily 0900 EPS (min, max, mean), Rainfall (total), END |
| DRM | Daily Midnight Rainfall (total), START |
| DR9 | Daily 0900 Rainfall (total), END |

## Point Time Series Water Quality (TSWQ)

| PWQ | Point Water Quality- as measured EC, Temperature, pH, Turbidity, Instantaneous Stage and Discharge |
| --- | --- |

## Periodic Time Series Water Quality Extracts

| DET | Daily Midnight EC (min, max, mean), Temperature (min, max, mean), START |
| --- | --- |
| DPT | Daily Midnight pH (min, max, mean), Temperature (min, max, mean), START |
| DTU | Daily Midnight Turbidity (min, max, mean), START |

## Descrete Water Quality Sample Data

| WQX | Extract containing water quality Samples and Results Data |
| --- | --- |

Column headings from WQX extract:

| STATION | unique Site indentifier |
| --- | --- |
| STATION NAME | descriptive name for the station |
| SAMPNUM | sample identifier |
| BOTTLE | the bottle number of the sample |
| SAMPTYPE | the type of sample |
| PROJECT1 | project code to be associated with this sample |
| SRCSAMP | code describing the source of the sample |
| COLLSAMP | code describing the organisation which collected the sample |
| COLLMETH | code describing the collection method |
| PRESMETH1 | Preservation method used for this sample |
| LABREF | laboratory reference code |
| LABSAMPREF | sample reference supplied by the laboratory |
| INPUTSRC | code describing the source of the sample |

# Time Series Data Parameters, Units

| Name | Description | Variable Number |
| --- | --- | --- |
| Stage | m GHt (metres Gauge Height) | 100.00 |
| Discharge | m3/sec (cubic metres per second, aka 'cumecs') | 140.00 |
| Volume | ML (megalitres) | 151.00 |
| Rainfall | mm (millimetres) | 10.00 |
| Electrical Conductivity (EC) | μS/cm (micro Siemens per centimetre) | 2010.00 |
| Temperature | °C (degrees Celsius) | 2080.00 |
| pH | pH (pH units) | 2100.00 |
| Turbidity | NTU (Nephelometric Turbidity Units) | 2030.00 |
| Bore Water Level | m (metres) | 110.00 |
| Groundwater Elevation AHD | m (metres) | 111.00 |
| Artesian Equivalent Potentiometric Surface (EPS) | m (metres) | 113.00 |
| Artesian Equivalent Potentiometric Elevation AHD (EPS) | m (metres) | 114.00 |

# Copyright

This publication has been compiled by Divisional Support (Water), Department of Regional Development, Manufacturing and Water

© State of Queensland, 2021.

The Queensland Government supports and encourages the dissemination and exchange of its information. The copyright in this publication is licensed under a Creative Commons Attribution 4.0 International (CC BY 4.0) licence.

Under this licence you are free, without having to seek our permission, to use this publication in accordance with the licence terms.

You must keep intact the copyright notice and attribute the State of Queensland as the source of the publication.

Note: Some content in this publication may have different terms as indicated.

For more information on this licence, visit https://creativecommons.org/licenses/by/4.0/.

# Disclaimer

The information contained herein is subject to change without notice. The Queensland Government shall not be liable for technical or other errors or omissions contained herein. The reader/user accepts all risks and responsibility for losses, damages, costs and other consequences resulting directly or indirectly from using this information.