DNRME Water Monitoring Information System

Hydrology Archive Data Information Sheet

Content

Time Series water monitoring data are supplied as compressed ZIP archives. The ZIP archives are:

| SDVR_[MMMYYYY].ZIP | Stage, Discharge, Volume, Rainfall |
| --- | --- |
| TSQW_[MMMYYYY].ZIP | Time Series Water Quality |
| RAIN_[MMMYYYY].ZIP | Time Series Rainfall |
| ART_[MMMYYYY].ZIP | Artesian Level, Rainfall |
| SUBART_[MMMYYYY].ZIP | Groundwater Level, Rainfall |

Metadata and File Types

- Metadata Files

| Surface Water Monitoring Network Index | WMP016_8.0_Stream_Gauging_Station_Network_2019_20.pdf |
| --- | --- |
| Groundwater Monitoring Network Index | WMP017_7.1_Groundwater_Network_2019_20.pdf |
| Spatial Site Data | DNRM_SITE_[YYYYMMDD].CSV |
| Quality Code Mapping | DNRM_DA_QUALITY.CSV |
| [GS].SR.TXT | Individual Gauging Station Summary Report |

CSV Data File Type: [GS].[TYPE].CSV

[GS]  Abbreviation for "Gauging Station", also referred to as a "Site".

[SS]  Abbreviation for "Subartesian Station", also referred to as a "Site".

[AS]  Abbreviation for "Artesian Station", also referred to as a "Site".

- Point Hydrologic Data

| PS | Point Stage (i.e. 'Gauge Height') as measured |
| --- | --- |
| PD | Point Discharge as stored * |
| SS | Point Stage (i.e. 'Subartesian Water Level') as measured |
| AS | Point Stage (i.e. 'Artesian EPS') as measured |
| PR | Point Rainfall as measured |

- Periodic Hydrologic Extracts (nb. START/ END indicates the period to which each record refers)

| DM | [GS] Daily Midnight Stage (min, max, mean), Discharge (min, max, mean), Volume (total), Rainfall (total),START |
| --- | --- |
| SM | [SS] Daily Midnight Water Level (min, max, mean), Rainfall (total),START |
| AM | [AS] Daily Midnight EPS (min, max, mean), Rainfall (total),START |
| D9 | [GS] Daily 0900 Stage (min, max, mean), Discharge (min, max, mean), Volume (total), Rainfall (total), END |
| S9 | [SS] Daily 0900 Water Level (min, max, mean), Rainfall (total), END |
| A9 | [AS] Daily 0900 EPS (min, max, mean), Rainfall (total), END |
| DRM | Daily Midnight Rainfall (total), START |
| DR9 | Daily 0900 Rainfall (total), END |

- Point Time Series Water Quality (TSWQ)

| PWQ | Point Water Quality- as measured EC, Temperature, pH, Turbidity, Instantaneous Stage and Discharge |
| --- | --- |

- Periodic Time Series Water Quality Extracts

| DET | Daily Midnight EC (min, max, mean), Temperature (min, max, mean), START |
| --- | --- |
| DPT | Daily Midnight pH (min, max, mean), Temperature (min, max, mean), START |
| DTU | Daily Midnight Turbidity (min, max, mean), START |

- Time Series Data Parameters, Units

| Name | Description | Variable Number |
| --- | --- | --- |
| Stage | m GHt (metres Gauge Height) | 100.00 |
| Discharge | m3/sec (cubic metres per second, aka 'cumecs') | 140.00 |
| Volume | ML (megalitres) | 151.00 |
| Rainfall | mm (millimetres) | 10.00 |
| Electrical Conductivity (EC) | μS/cm (micro Siemens per centimetre) | 2010.00 |
| Temperature | °C (degrees Celsius) | 2080.00 |
| pH | pH (pH units) | 2100.00 |
| Turbidity | NTU (Nephelometric Turbidity Units) | 2030.00 |
| Bore Water Level | m (metres) | 110.00 |
| Groundwater Elevation AHD | m (metres) | 111.00 |
| Artesian Equivalent Potentiometric Surface (EPS) | m (metres) | 113.00 |
| Artesian Equivalent Potentiometric Elevation AHD (EPS) | m (metres) | 114.00 |